Lab 4b

Name____________________________________Date____________Time___________

Problem 5.78—Engineering Fluid Mechanics (Crowe, Elger and Robertson)

A suction device is being designed based on the venture principle to lift objects submerged in water.  The operating water temperature is 15o C.  The suction cup is located 1 m below the water surface, and the venture throat is located 1 m above the water. The atmosphere pressure is 100 kPa. The ratio of the throat area to the exit area is ¼, and the exit area is 0.001 m2.  The area of the suction cup is 0.1 m2.

- Find the velocity of the water at the exit for the maximum lift condition.

- Find the discharge through the system for maximum lift condition.

- Find the maximum load the suction can support

Pv 15C= 1700 Pa	ρ=999 kg/m3		Patm=100 KPa	Twater=15o C

Ae=10-3 m2		Δh= 2 m	As= 0.1 m2

a) Write the Bernoulli equation from the throat to the exit of the channel.  The pressure at the throat is equal to the Pv.

Also VtAt=VeAe

Vt=VeAe/At=___Ve

Substitute the value into the Bernoulli equation written above and solve for Ve.

Ve=________m/s

b) Q= Ve*Ae=________*________=________m3/s

c)

Psuction= Pt + γΔh

Psuction	=_____________+__________*__________=_____________Pa

The pressure surrounding the suction (ΔP) will be the difference between Pwater and

Psuction.

Pwater= Patm + γ * 1 m= _________Pa

ΔP= Pwater - Psuction= ________ Pa -_____________Pa = ___________Pa

Max Lift

Flift= ΔP * As =_________N/m2 *_________m2= ____________N