## Application of the ArcGIS Data Model to an Engineering Study

TxDOT Geodatabase

Prepared by Tanya Hoogerwerf

May 31, 2001

The data used in the TxDOT geodatabase result from a digital floodplain analysis conducted by David James Anderson at a highway river crossing in Pecan Bayou, Texas, for the Texas Department of Transportation (TxDOT).  In this engineering study, GIS was used to facilitate hydrologic and hydraulic modeling of the study area.

The study area lies directly downstream of Lake Brownwood, which serves as an uncontrolled water supply discharging to Pecan Bayou as it proceeds to the Brazos River.  The watershed, with an area of 515.2 sq mi, is delineated from Lake Brownwood to the USGS gauging station (downstream of Pecan Bayou). The site contains a precipitation gauge and a USGS flow gauge, which records flow at 15-minute intervals.

A hydrologic analysis was conducted to calculate the quantity of runoff that was generated for each rainfall event that occurs in a particular watershed; and hydrologic modeling was applied to determine the water surface profiles that can be expected from the runoff calculated with hydrologic modeling.

The first step needed to develop the digital spatial database before performing the modeling is data collection.  The spatial data gathered for this study includes a 30-meter digital elevation model (DEM), contour data in MicroStation format, stream networks in National Hydrography Dataset (NHD) format, site-specific field data, curve number grids, road coverages, USGS flow and stage data and precipitation data.  A combination of the 30-meter DEM and elevation contour data can be used to produce a digital terrain model (DTM).

The following table outlines the steps used in this analysis:

| STEP | Methodology | Modeling Program/Preprocessor and/or GIS | Description |
| --- | --- | --- | --- |
| 1. | Terrain Data Development | CRWR-PrePro, ArcView | Terrain development for hydrologic analysis using a 30-meter USGS digital elevation model (DEM). In this step the stream and watershed networks are delineated. Outlets are added at user-defined points, such as at structure locations. |
|  |  | CRWR-FloodMap, ArcView | Terrain development for hydraulic analysis using triangular irregular networks (TINs). |
| 2. | GIS-Based Hydrologic Parameter Extraction | CRWR-PrePro, ArcView | CRWR-PrePro builds a spatially variable hydrologic parameter extraction for export to HEC-HMS. |
| 3. | GIS-Based Hydraulic Parameter Extraction | HEC-GeoRas, ArcView | Topographic information extraction from a terrain TIN for input into HEC-RAS. |
| 4. | Hydrologic Modeling | HEC-HMS | This lumped model produces a discharge hydrograph based on basin data extracted by CRWR-PrePro (step 2.). |
| 5. | Hydraulic Modeling | HEC-RAS | This one-dimensional hydraulic model generates a waters surface profile for a design storm (using input from step 3.). Flows from hydrologic modeling (step 4.) are used in hydraulic modeling. |
| 6. | Floodplain Delineation | HEC-GeoRas, ArcView | This model imports waters surface profiles (as cross sections, centerlines, and overbank geometric data) from HEC-RAS and displays the floodplain in GIS. |

The TxDOT Geodatabase contains sixteen features classes.  These have all been prepared during the process described above.  In addition, a time series has been added.  The time series contains precipitation, lake discharge and flow data for a December 1991 storm event.  These are the data used in modeling the Pecan Bayou watershed.

# Drainage

| Feature Class Name | Description | Preparation |
| --- | --- | --- |
| Watershed | DEM-based watersheds | From Step 1 above |
| DrainagePath | DEM-based drainage paths | From Step 1 above |
| DrainagePoint | DEM-based outlets (this includes user-defined outlets) | From Step 1 above |

# Network

| Feature Class Name | Description | Preparation |
| --- | --- | --- |
| HydroEdge | National Hydrography Dataset (NHD) derived feature | Edited and merged route.reach and route.drain |
| HydroNetwork | Stream Network | Network created in ArcMap 8.1 |
| HydroJunction | Stream Junction Points | Merged HydroJunctions created by ArcMap 8.1 and Outlets (from Step 1) that have been snapped to the stream network |
| HydroNetwork_ Junctions | Stream Network Junctions | Network created in ArcMap 8.1 |
| SchematicNode | Schematic Points | Created by CRWR PrePro |
| SchematicLink | Schematic Lines | Created by CRWR PrePro |

# Hydrography

| Feature Class Name | Description | Preparation |
| --- | --- | --- |
| Hydroline | National Hydrography Dataset (NHD) derived feature | Edited and merged route.reach and route.drain |
| Waterbody | National Hydrography Dataset (NHD) derived feature | None |
| Structure | Locations of highway drainage structures and critical points of analysis | This shapefile includes additional outlets added to the stream grid. Three bridges are also included in these outlets. |
| MonitoringPoint | USGS Flow Gauging Station | This shapefile was created from the USGS tabular data. |

# Channel

| Feature Class Name | Description | Preparation |
| --- | --- | --- |
| ProfileLine | Channel Geometry | ProfileLine includes the digital stream derived from Microstation contours and bounding locations. |
| CrossSection | Channel Geometry | These data were provided by the Corps Ft. Worth District, but edited and interpolated to adequately define the stream centerline. |

# Time Series

| Feature Class Name | Description | Preparation |
| --- | --- | --- |
| TimeSeries | Historical flow and precipitation for December 1991 storm event. | The time series was created in Excel, then imported into ArcMap 8.1. |

For further information on this project, please refer to David Anderson’s webpage:

http://www.ce.utexas.edu/stu/andersdj/home.html